SmartCar

An AI-Powered Autonomous Vehicle System

with Real-Time IoT Control

A Project Report

Submitted in partial fulfillment of the requirements for the degree of

Bachelor of Technology

Submitted by:

Prakhar Sharma (23EEAEC034)

Vidhi Jangid (23EEAEC027)

Under the guidance of:

Dr. Rajesh Kumar Raj

Department of Electronics and Communication Engineering

Engineering College Ajmer, Bikaner Technical University

Academic Year 2025 - 2026

Candidate's Declaration

I hereby declare that the work, which is being presented in the Project, entitled "SmartCar: An AI-Powered Autonomous Vehicle System with Real-Time IoT Control" in partial fulfilment for the award of Degree of "Bachelor of Technology" in Electronics and Communication Engineering and submitted to the Department of Electronics and Communication Engineering, Engineering College Ajmer, Bikaner Technical University is a record of my own investigations carried under the Guidance of Dr. Rajesh Kumar Raj, Department of Electronics and Communication Engineering, Engineering College Ajmer.

I have not submitted the matter presented in this report anywhere for the award of any other Degree.

1.  Prakhar Sharma (23EEAEC034)

2.  Vidhi Jangid (23EEAEC027)

Electronics and Communication Engineering,

Engineering College Ajmer

Acknowledgement

The project report on "SmartCar: An AI-Powered Autonomous Vehicle System with Real-Time IoT Control" is outcome of guidance, moral support and devotion bestowed on us throughout our work. For this, we acknowledge and express our profound sense of gratitude and thanks to everybody who have been a source of inspiration during the project preparation.

We offer our sincere phrases of thanks with to our Project Guide, Sh. Harish Kumar Sharma without whose support and guidance it would not have been possible for this Project to have materialized and taken a concrete shape.

We are thankful to the Department of Electronics and Communication Engineering, Govt. Engineering College Ajmer for providing the necessary resources and academic environment required for the successful completion of this project.

1. Prakhar Sharma (23EEAEC034)

2. Vidhi Jangid (23EEAEC027)

# Table of Contents

1.  Abstract

2.  Introduction

3.  Literature Review

4.  System Architecture

5.  Hardware Components and Specifications

6.  Software Architecture

7.  Features and Working

8.  Circuit Diagram and Connections

9.  AI and Machine Learning Modules

10. IoT and Cloud Integration

11. Testing and Results

12. Comparison with Industry Solutions

13. Future Prospects

14. Conclusion

15. References

# 1. Abstract

This project presents the design and implementation of SmartCar, an AI-powered autonomous vehicle system built on a Raspberry Pi 3 platform with 1GB RAM. The system demonstrates real-time computer vision capabilities including traffic light detection using HSV color segmentation, PID-based lane following, collision prediction with automatic braking, pedestrian detection with movement prediction, road condition analysis, smart auto-parking, and convoy (follow-me) mode.

The vehicle can be controlled through three independent channels: a local web application over WiFi, a Telegram bot for remote control with emergency SOS capabilities, and an MQTT-based cloud relay for internet control from anywhere in the world. The system processes camera frames at the edge (on-device) with sub-100ms latency, demonstrating that intelligent vehicle systems can be implemented on ultra-low-cost hardware (total BOM under INR 5,000) without relying on cloud computing for decision-making.

The project also includes driver behavior analysis with safety scoring, accident detection with automatic Telegram alerts, and comprehensive session logging for post-drive analysis.

Keywords: Autonomous Vehicle, Raspberry Pi, Computer Vision, PID Control, IoT, MQTT, Telegram Bot, Edge AI, Collision Detection, Lane Following, Traffic Light Detection

# 2. Introduction

## 2.1 Background

The automotive industry is undergoing a paradigm shift with the advent of autonomous driving technology. Companies like Tesla, Waymo, and Cruise have invested billions of dollars in developing self-driving vehicles equipped with advanced sensors including LiDAR, radar arrays, and multi-camera systems powered by high-performance GPU computing platforms. While these systems achieve remarkable results, they remain inaccessible to the average consumer and prohibitively expensive for educational and research purposes.

This project explores the question: Can meaningful autonomous driving capabilities be achieved on ultra-low-cost hardware using only a single camera and open-source software?

## 2.2 Problem Statement

Current autonomous vehicle development requires expensive hardware (LiDAR sensors costing $5,000+, high-performance computing units at $10,000+) and proprietary software. There is a need for a low-cost, accessible platform that demonstrates core autonomous driving principles for educational purposes and proof-of-concept development.

## 2.3 Objectives

- Design and build an autonomous vehicle using only a Raspberry Pi 3 and a single camera.

- Implement real-time traffic light detection using computer vision.

- Develop PID-controlled lane following with black line tracking.

- Create AI-based collision prediction and automatic emergency braking.

- Implement pedestrian detection with movement prediction.

- Build smart auto-parking using visual markers.

- Develop multi-channel remote control (WiFi, Telegram, Cloud).

- Create driver behavior analysis and safety scoring system.

- Implement accident detection with automatic emergency alerts.

## 2.4 Scope

The project covers the complete pipeline from hardware assembly and wiring to software development, AI integration, IoT connectivity, and cloud deployment. The system operates as a functional prototype demonstrating 6 driving modes, 6 AI features, and 3 control channels.

# 3. Literature Review

## 3.1 Autonomous Driving - Industry Landscape

The Society of Automotive Engineers (SAE) defines 6 levels of driving automation (Level 0 to Level 5). Current commercial systems like Tesla Autopilot operate at Level 2 to 3, requiring driver supervision. Full Level 5 autonomy remains an unsolved challenge. Our SmartCar operates at Level 2 equivalent in controlled environments, handling lane keeping, traffic signal response, and emergency braking.

## 3.2 Computer Vision in Autonomous Vehicles

OpenCV (Open Source Computer Vision Library) provides efficient implementations of image processing algorithms. HSV color space segmentation has been proven effective for traffic light detection in controlled lighting conditions (Diaz et al., 2015). PID controllers remain the standard for lane-keeping assist systems due to their simplicity and effectiveness (Zhao et al., 2017).

## 3.3 Edge AI vs Cloud AI

Edge computing processes data locally on the device rather than sending it to cloud servers. For autonomous vehicles, this is critical because network latency (100 to 500ms for cloud roundtrip) is unacceptable for safety-critical decisions like emergency braking. Our system processes all AI inference at the edge with sub-100ms latency.

## 3.4 IoT in Automotive

MQTT (Message Queuing Telemetry Transport) is the de-facto standard for IoT communication, offering lightweight publish/subscribe messaging ideal for constrained devices. Vehicle-to-Everything (V2X) communication enables vehicles to share data with other vehicles, infrastructure, and cloud services.

# 4. System Architecture

## 4.1 High-Level Architecture

The SmartCar system is organized into five distinct layers as shown below:

User Interface Layer
      Flask Web App  |  Netlify Cloud App  |  Telegram Bot
                           |
    Application Layer
      Flask + SocketIO  |  MQTT Client  |  Telegram Bot API
                           |
    AI / Computer Vision Layer
      Collision  |  Pedestrian  |  Traffic  |  Line Follower  |  Road  |  Convoy
                           |
    Driver / Control Layer
      Motor Controller (GPIO + PWM)  |  Camera Stream  |  LED & Buzzer
                           |
    Hardware Layer
      Raspberry Pi 3  |  L298N  |  4x BO Motors  |  Pi Camera  |  LED  |  Buzzer

Figure 1: Five-layer system architecture of SmartCar

## 4.2 Data Flow

A single camera frame captured at 256 by 192 pixels is split and processed simultaneously by multiple AI modules. The top half of the frame feeds into traffic light detection, the bottom half drives the line following algorithm, and the full frame is used for collision prediction, pedestrian detection, road condition analysis, and accident detection. All AI decisions are routed to the motor controller, buzzer, LED indicators, and Telegram notification systems.

Figure 2: Camera frame processing pipeline - single frame, multiple AI modules

# 5. Hardware Components and Specifications

## 5.1 Component List

| S.No | Component | Specification | Purpose | Qty |
| --- | --- | --- | --- | --- |
| 1 | Raspberry Pi 3 Model B | 1.2GHz Quad-Core ARM, 1GB RAM, WiFi, BT | Main processing unit | 1 |
| 2 | Pi Camera Module v1.3 | OV5647, 5MP, 1080p at 30fps, CSI interface | Vision input for all AI | 1 |
| 3 | L298N Motor Driver | Dual H-Bridge, 5 to 35V, 2A per channel | Motor speed and direction | 1 |
| 4 | BO Motors (Geared DC) | 3 to 12V DC, 100 to 200 RPM | Wheel drive (differential) | 4 |
| 5 | LM2596 Buck Converter | DC-DC Step Down, 1.25 to 35V adjustable | Regulated 5.1V for Pi | 1 |
| 6 | Li-ion Battery Pack | 3.7V cells, 7.4V to 12V configuration | Power source | 1 |
| 7 | White LED Plate | 5V, with 220 ohm resistor | Headlight or indicator | 1 |
| 8 | Active Buzzer | 3 to 5V, 85dB | Horn, alert, and alarm | 1 |
| 9 | 4WD Car Chassis | Acrylic or metal platform with motor mounts | Vehicle body | 1 |
| 10 | Jumper Wires | Male-Female, Male-Male | Connections | ~20 |
| 11 | Resistors | 220 ohm | LED current limiting | 1 |

Table 1: Bill of Materials (BOM) - Total estimated cost: INR 4,000 to 5,000

## 5.2 Raspberry Pi 3 - Key Specifications

| Parameter | Value |
| --- | --- |
| Processor | Broadcom BCM2837, 64-bit ARMv8, Quad-core at 1.2GHz |
| RAM | 1GB LPDDR2 |
| GPIO | 40-pin header, 26 GPIO pins |
| WiFi | 802.11n 2.4GHz |
| Bluetooth | Bluetooth 4.1, BLE |
| Camera Interface | CSI (Camera Serial Interface) |
| OS | Raspberry Pi OS (Debian Bullseye) |
| Power | 5V / 2.5A via micro-USB or GPIO |

Table 2: Raspberry Pi 3 Model B specifications

# 6. Software Architecture

## 6.1 Technology Stack

| Layer | Technology | Purpose |
| --- | --- | --- |
| Programming Language | Python 3.9 | All backend logic, AI, and hardware control |
| Web Framework | Flask + Flask-SocketIO | Local web server with real-time communication |
| Computer Vision | OpenCV 4.x | Image processing, HSV segmentation, contours |
| Camera Library | PiCamera (legacy) | Raspberry Pi Camera interface |
| IoT Protocol | MQTT (paho-mqtt) | Cloud relay via broker.emqx.io |
| Bot Framework | python-telegram-bot | Telegram bot for remote control |
| Frontend | HTML5, CSS3, JavaScript | Web control panel (deployed on Netlify) |
| GPIO Control | RPi.GPIO | Hardware pin control |
| Numerical Computing | NumPy | Array operations for image processing |

Table 3: Software technology stack

## 6.2 File Structure

SmartCar/
  |-- main.py                 # All-in-one: Flask + Telegram + MQTT + AI
  |-- config.py               # Pin definitions, constants, thresholds
  |-- motor.py                # L298N motor driver controller
  |-- camera.py               # Camera capture (PiCamera + OpenCV fallback)
  |-- line_follower.py        # PID-based lane following
  |-- traffic_detector.py     # Traffic light HSV detection
  |-- smart_park.py           # Auto-parking with marker detection
  |-- ai_features.py          # Collision, Pedestrian, Road, Driver, Convoy
  |-- session_log.py          # Event logging with timestamps
  |-- templates/
  |   +-- index.html          # Local web control panel
  |-- frontend/
  |   +-- index.html          # Cloud web panel (deployed on Netlify)
  +-- logs/
      +-- session_*.log       # Session event logs

Figure 3: Project file structure

# 7. Features and Working

## 7.1 Driving Modes

### Mode 1: Manual Control

The user controls the car in real-time through the web application or Telegram bot. Controls include forward, backward, left, right, and stop. Speed is adjustable via a slider (0 to 100 percent). The driver behavior analysis module tracks all commands and generates a safety score.

### Mode 2: Line Following

The car autonomously follows a black line on a white surface using PID (Proportional-Integral-Derivative) control. The camera captures the bottom half of the frame, converts it to binary (the black line becomes white pixels), finds the line centroid, and calculates the error from the frame center. The PID controller adjusts left and right motor speeds to minimize this error, providing smooth trajectory tracking.

The PID equation used is as follows:

correction = Kp x error + Ki x sum(error) + Kd x delta(error)

Left Speed = Base Speed + correction

Right Speed = Base Speed - correction

Where Kp = 0.4, Ki = 0.0, Kd = 0.2

### Mode 3: Traffic Light Detection

The car detects traffic light colors (red, green, yellow) using HSV color space segmentation on the top half of the camera frame. The system uses dual HSV ranges for red (which wraps around the hue spectrum at 0 and 180 degrees) and single ranges for green and yellow. Morphological operations (opening and closing) reduce noise. The car stops on red, proceeds on green, and slows down on yellow.

### Mode 4: Auto Pilot

This mode combines line following and traffic light detection. The car follows the lane while simultaneously monitoring for traffic signals. If a red light is detected, the car stops regardless of its line position. On green, the car resumes line following. This demonstrates multi-task computer vision on resource-constrained hardware.

### Mode 5: Smart Parking

The car autonomously parks between two green markers. The process follows a finite state machine (FSM) with four phases: Scanning (rotating to find markers), Aligning (centering between markers), Reversing (backing into the spot), and Parked (final state with confirmation buzzer). The LED and buzzer blink at 1-second intervals during parking mode.

### Mode 6: Convoy (Follow Me)

The car follows a colored object selected by the user via Telegram. Available colors include red, blue, green, yellow, orange, pink, and white. The system uses HSV color tracking with a PD (Proportional-Derivative) controller for smooth steering. Distance is estimated from the contour area, where a larger area means a closer object, which triggers speed reduction.

## 7.2 AI Safety Features (Active in All Modes)

### Collision Prediction and Auto-Brake

The system tracks the growth rate of objects in the camera center region across consecutive frames. If an object bounding area grows consistently for 4 or more frames (indicating rapid approach), the system calculates Time-to-Collision (TTC) and triggers automatic emergency braking. An instant alert with 2 photos is sent to the Telegram bot.

### Pedestrian Detection and Movement Prediction

Using frame differencing with aspect ratio filtering (height greater than width, characteristic of human silhouettes), the system detects moving objects in the upper-center camera region. Centroid tracking across frames predicts the pedestrian direction (left, right, approaching, or stationary). If approaching, the car auto-stops and sends a Telegram alert.

### Accident Detection and SOS

The system maintains a running baseline of normal frame-to-frame changes during driving. An accident is detected when the frame change exceeds 3 times the baseline while the motor was running for more than 1 second. Upon detection, the motor stops immediately, the LED and buzzer flash rapidly, 5 photos are captured at 2-second intervals, and all photos are sent to the owner Telegram account with an ACCIDENT DETECTED alert.

### Road Condition Analysis

The bottom strip of the camera frame is analyzed using the Laplacian variance method. Smooth surfaces produce low variance, normal surfaces produce medium variance, and rough surfaces produce high variance. The system automatically reduces motor speed proportionally: 100 percent for smooth, 80 percent for normal, and 50 percent for rough surfaces. A 10-frame running average prevents flickering between states.

### Driver Behavior Analysis

During manual driving, the system logs all motor commands with timestamps and calculates a driving safety score (0 to 100). Deductions are made for sharp turns (direction change within 0.5 seconds), hard brakes (stop from high speed within 0.3 seconds), and sudden speed changes (more than 25 percent change). The score and statistics are viewable via the Telegram bot.

# 8. Circuit Diagram and Connections

## 8.1 GPIO Pin Mapping

| Component | GPIO Pin | Physical Pin | Function |
| --- | --- | --- | --- |
| L298N ENA | GPIO 25 | Pin 22 | Left motor enable (PWM) |
| L298N IN1 | GPIO 24 | Pin 18 | Left motor direction A |
| L298N IN2 | GPIO 23 | Pin 16 | Left motor direction B |
| L298N ENB | GPIO 12 | Pin 32 | Right motor enable (PWM) |
| L298N IN3 | GPIO 17 | Pin 11 | Right motor direction A |
| L298N IN4 | GPIO 27 | Pin 13 | Right motor direction B |
| LED Plate | GPIO 5 | Pin 29 | Headlight or indicator |
| Buzzer | GPIO 19 | Pin 35 | Horn and alerts |

Table 4: GPIO pin assignments

## 8.2 Wiring Diagram

BATTERY (7.4V to 12V)
                          |
            +-------------+-------------+
            |                           |
       L298N +12V                  LM2596 IN+
       L298N GND ---+---- LM2596 IN-
                    |
               Pi GND (Pin 6)     LM2596 OUT+ ---> Pi 5V (Pin 2)
                                  LM2596 OUT- ---> Pi GND (Pin 6)

  L298N Motor Outputs:
      OUT1 + OUT2 ----> Left 2 Motors (parallel)
      OUT3 + OUT4 ----> Right 2 Motors (parallel)

  L298N Control (from Pi GPIO):
      ENA <---- GPIO 25 (Pin 22)     ENB <---- GPIO 12 (Pin 32)
      IN1 <---- GPIO 24 (Pin 18)     IN3 <---- GPIO 17 (Pin 11)
      IN2 <---- GPIO 23 (Pin 16)     IN4 <---- GPIO 27 (Pin 13)

  Peripherals:
      GPIO 5 (Pin 29) --[220 ohm]-- LED (+) ---- GND (Pin 30)
      GPIO 19 (Pin 35) ------------- Buzzer (+) -- GND (Pin 39)

  Camera:
      Pi Camera v1.3 ---[Ribbon Cable]---> CSI Port

Figure 4: Complete wiring diagram

## 8.3 Differential Drive System

The SmartCar uses a 4-wheel differential drive configuration. Two left motors are wired in parallel (connected to OUT1 and OUT2 of L298N), and two right motors are wired in parallel (connected to OUT3 and OUT4). Steering is achieved by varying the speed difference between left and right motor pairs.

| Movement | Left Motors | Right Motors |
| --- | --- | --- |
| Forward | Forward (HIGH) | Forward (HIGH) |
| Backward | Backward (HIGH) | Backward (HIGH) |
| Turn Left | Backward | Forward |
| Turn Right | Forward | Backward |
| Soft Left | Forward (slow) | Forward (fast) |
| Stop | OFF | OFF |

Table 5: Differential drive motor states

# 9. AI and Machine Learning Modules

## 9.1 Processing Strategy for 1GB RAM

Running multiple AI modules on a Raspberry Pi 3 with only 1GB RAM requires careful optimization. The following strategies were employed:

- Resolution downscaling: All AI processing runs at 160 by 120 pixels (camera captures at 256 by 192).

- Frame skipping: AI processes every 3rd frame, yielding approximately 7 FPS for AI while the camera runs at 10 FPS.

- ROI selection: Each module processes only relevant image regions (top half, bottom half, center, etc.).

- No deep learning: All algorithms use classical computer vision (OpenCV), avoiding memory-hungry neural networks.

- Deque buffers: Fixed-size circular buffers prevent unbounded memory growth.

## 9.2 Algorithm Summary

| Module | Algorithm | Input Region | Resolution | Latency |
| --- | --- | --- | --- | --- |
| Traffic Detection | HSV Segmentation + Morphology | Top half | 160x120 | ~3ms |
| Line Following | Thresholding + Contour + PID | Bottom half | 256x192 | ~5ms |
| Collision Prediction | Canny Edges + Area Growth | Center region | 160x120 | ~4ms |
| Pedestrian Detection | Frame Diff + Aspect Filter | Upper center | 160x120 | ~3ms |
| Road Condition | Laplacian Variance | Bottom strip | 160x120 | ~1ms |
| Accident Detection | Frame Diff Spike + Baseline | Full frame | 80x60 | ~1ms |
| Convoy Following | HSV Tracking + PD Control | Full frame | 160x120 | ~3ms |

Table 6: AI module performance characteristics

## 9.3 False Positive Prevention

| Module | Prevention Method |
| --- | --- |
| Collision | Requires 4 consecutive danger frames and minimum object area threshold |
| Pedestrian | Aspect ratio filter (h > w), 3-frame confirmation, 5-frame hysteresis |
| Accident | Running baseline (20 frames), spike must exceed 3x baseline, motor running over 1s |
| Road Condition | 10-frame running average prevents flickering between states |

Table 7: False positive prevention strategies

# 10. IoT and Cloud Integration

## 10.1 Three Control Channels

| Property | Channel 1: Local WiFi | Channel 2: Telegram Bot | Channel 3: MQTT Cloud |
| --- | --- | --- | --- |
| Protocol | HTTP + WebSocket | HTTPS (Telegram API) | MQTT over WSS |
| Framework | Flask + SocketIO | python-telegram-bot | paho-mqtt + Netlify |
| Latency | Less than 10ms | 100 to 300ms | 50 to 200ms |
| Internet Required | No | Yes | Yes |

Table: Control channel comparison

## 10.2 MQTT Topic Architecture

| Topic | Direction | Payload |
| --- | --- | --- |
| smartcar2027/cmd | Frontend to Pi | forward, backward, left, right, stop |
| smartcar2027/speed | Frontend to Pi | 0 to 100 (integer) |
| smartcar2027/fashion | Frontend to Pi | manual, line_follow, traffic, autopilot, etc. |
| smartcar2027/led | Frontend to Pi | state: true or false (JSON) |
| smartcar2027/cam | Pi to Frontend | JPEG binary (approximately 5KB per frame) |
| smartcar2027/status | Pi to Frontend | JSON with mode, speed, and AI status |

Table 8: MQTT topic definitions

## 10.3 Accident SOS Pipeline

When an accident is detected (frame change exceeding 3 times the running baseline), the following pipeline is triggered: First, all motors are stopped immediately. Then the LED begins flashing and the buzzer sounds. Next, 5 photographs are captured at 2-second intervals. Finally, all photographs are transmitted to the owner via Telegram along with an ACCIDENT DETECTED alert message.

# 11. Testing and Results

## 11.1 Test Scenarios

| Test | Scenario | Expected Result | Status |
| --- | --- | --- | --- |
| Motor Control | Forward/Back/Left/Right via webapp | Motors respond correctly | PASS |
| Camera Stream | Live MJPEG feed on phone browser | Smooth ~10 FPS stream | PASS |
| Line Following | Black tape on white floor with curves | Car follows line smoothly | PASS |
| Traffic Light | Red/Green paper shown to camera | Stops on red, goes on green | PASS |
| Auto Pilot | Line track with traffic signals | Follows line and obeys signals | PASS |
| Smart Parking | Two green markers on floor | Car scans, aligns, reverses in | PASS |
| Convoy Mode | Red ball moved in front of car | Car follows the ball | PASS |
| Collision Brake | Object moved rapidly toward camera | Auto-brakes and Telegram alert | PASS |
| SOS Accident | Car bumped into wall | 5 photos sent to Telegram | PASS |
| Telegram Control | Drive car from Telegram buttons | Motors respond to button presses | PASS |
| Cloud Control | Control from Netlify webapp | MQTT commands reach Pi | PASS |
| Session Logging | Drive session with multiple events | All events logged with timestamps | PASS |
| Auto-Start | Power on Pi without SSH | SmartCar starts automatically | PASS |

Table 9: Test results summary - all 13 tests passed

## 11.2 Performance Metrics

| Metric | Value |
| --- | --- |
| Frame rate (camera) | 10 FPS |
| AI processing rate | Approximately 7 FPS (every 3rd frame) |
| Motor response time (local WiFi) | Less than 10ms |
| Motor response time (Telegram) | 100 to 300ms |
| Motor response time (MQTT cloud) | 50 to 200ms |
| Collision detection accuracy | Approximately 85% (controlled environment) |
| Traffic light detection accuracy | Approximately 90% (good lighting) |
| CPU usage (all features active) | 60 to 75% |
| RAM usage | 400 to 500MB of 1GB |
| Total system boot time | Approximately 30 seconds |

Table 10: System performance metrics

# 12. Comparison with Industry Solutions

| Parameter | Tesla Autopilot | Waymo | SmartCar (Ours) |
| --- | --- | --- | --- |
| Hardware Cost | ~$50,000+ | ~$150,000+ | ~$60 (INR 5,000) |
| Computing | Tesla FSD Chip (144 TOPS) | Custom TPU + Intel Xeon | RPi 3 (1.2GHz ARM) |
| Cameras | 8 cameras | 29 cameras | 1 camera |
| LiDAR | None (vision only) | 4 to 5 LiDAR units | None (vision only) |
| Internet Required | Yes (maps, updates) | Yes | No (works offline) |
| Processing Location | Edge (on-car) | Edge + Cloud | Edge (on-car) |
| Remote Control | Tesla App | Waymo App | Web + Telegram + Cloud |
| Accident Alert | Emergency services call | Operations center | Telegram SOS + photos |
| Open Source | No | No | Yes (fully open source) |

Table 11: Comparison with commercial autonomous vehicle platforms

The SmartCar achieves SAE Level 2 equivalent autonomous capabilities on hardware costing less than 0.1 percent of commercial solutions, proving that intelligent vehicle systems can be democratized through edge computing and open-source software.

# 13. Future Prospects

## 13.1 Short-Term Enhancements

- GPS Module Integration: Adding a NEO-6M GPS module (INR 200) would enable real-time location tracking, geofencing, and precise accident location sharing via Telegram.

- Ultrasonic Sensors: HC-SR04 sensors (INR 50 each) would provide precise distance measurement for parking and obstacle avoidance, complementing the camera-based system.

- Speed Sign Detection: OCR-based speed limit sign recognition using Tesseract to automatically adjust vehicle speed.

- Voice Control: Integration of a USB microphone with speech recognition for hands-free vehicle control.

- Night Vision: IR LED array for camera-based operation in low-light conditions.

## 13.2 Medium-Term Developments

- V2V Communication: Multiple SmartCars communicating via MQTT. When one car detects a red light, it can inform the car behind it to begin braking. The MQTT infrastructure is already in place.

- TensorFlow Lite Integration: Deploying lightweight neural network models for more accurate object classification (pedestrian vs cyclist vs animal).

- SLAM (Simultaneous Localization and Mapping): Building a real-time map of the environment for autonomous navigation without predefined tracks.

- Swarm Robotics: Coordinating multiple vehicles for warehouse-style logistics applications.

- Predictive Maintenance: Monitoring motor current draw and camera quality degradation to predict component failures.

## 13.3 Long-Term Vision

- Full-Scale Vehicle Integration: Adapting the software architecture for real vehicles using more powerful compute platforms (NVIDIA Jetson) while maintaining the same modular design.

- Cloud Digital Twin: A real-time 3D simulation mirroring the physical vehicle state, enabling remote monitoring and virtual testing.

- Edge-Cloud Hybrid AI: Using edge computing for safety-critical decisions (under 10ms) while offloading complex scene understanding to cloud GPU clusters.

- Fleet Management: A centralized dashboard managing multiple SmartCar units for delivery, surveillance, or agricultural applications.

- Regulatory Compliance: Adapting the system to meet emerging autonomous vehicle regulations and safety standards.

# 14. Conclusion

This project successfully demonstrates that meaningful autonomous driving capabilities can be achieved on ultra-low-cost hardware using only a single camera and open-source software. The SmartCar system, built on a Raspberry Pi 3 with 1GB RAM, implements six driving modes (Manual, Line Following, Traffic Light Detection, Auto Pilot, Smart Parking, and Convoy), six AI safety features (Collision Prediction, Pedestrian Detection, Road Condition Analysis, Accident Detection, Driver Behavior Analysis, and Follow-Me Tracking), and three independent control channels (Local WiFi, Telegram Bot, and MQTT Cloud).

Key achievements of this project include:

- Real-time edge AI processing at approximately 7 FPS for safety-critical decisions with sub-100ms latency.

- Multi-task computer vision where a single camera frame is simultaneously processed for traffic signals, lane position, obstacles, pedestrians, and road conditions.

- True IoT integration allowing the vehicle to be monitored and controlled from anywhere in the world via the internet.

- Automatic emergency response where accident detection triggers instant alerts with photographic evidence to the owner phone.

- Total hardware cost under INR 5,000, which is approximately 0.1 percent of commercial autonomous vehicle sensor suites.

The project proves that the core principles of autonomous driving, namely perception, decision-making, and control, are accessible to students and researchers without requiring enterprise-grade hardware. The modular software architecture ensures that each component can be independently improved or replaced, making this an excellent foundation for further research and development in the field of intelligent transportation systems.

# 15. References

[1]  SAE International, "Taxonomy and Definitions for Terms Related to Driving Automation Systems for On-Road Motor Vehicles," SAE J3016, 2021.

[2]  Bradski, G. and Kaehler, A., "Learning OpenCV: Computer Vision with the OpenCV Library," O'Reilly Media, 2008.

[3]  Diaz, M. et al., "Traffic Light Detection using HSV Color Space," IEEE International Conference on Computer Vision, 2015.

[4]  Zhao, D. et al., "PID Controller Design for Autonomous Vehicle Lane Keeping," IEEE Transactions on Intelligent Transportation Systems, vol. 18, no. 4, 2017.

[5]  MQTT.org, "MQTT Version 5.0 - OASIS Standard," 2019.

[6]  Raspberry Pi Foundation, "Raspberry Pi 3 Model B - Technical Specifications," 2016.

[7]  OpenCV Documentation, "OpenCV-Python Tutorials," https://docs.opencv.org, 2024.

[8]  Upton, E. and Halfacree, G., "Raspberry Pi User Guide," 4th Edition, Wiley, 2020.

[9]  TensorFlow Lite, "Deploy Machine Learning Models on Mobile and IoT Devices," https://www.tensorflow.org/lite, 2024.

[10] python-telegram-bot Documentation, "python-telegram-bot v21.x," https://python-telegram-bot.org, 2024.